CHECKLIST TEMPLATE

# Dental office opening and closing checklist

Check this template against your local regulations and your own policies before you use it.

| Business |  | Location |  | Date |  |
| --- | --- | --- | --- | --- | --- |

# Opening checklist

Before the first patient

| Done | Task | Time | Initials |
| --- | --- | --- | --- |
| Equipment start-up |  |  |  |
| ☐ | Disarm the alarm and turn on the lights and heating or cooling |  |  |
| ☐ | Turn on the compressor and suction, and check for leaks or warning lights Drain the compressor if the maker requires it. |  |  |
| ☐ | Turn on the sterilizer and run any daily test cycle the maker requires Record the result in the sterilization log before any load is run. |  |  |
| ☐ | Turn on the X-ray units and imaging computers |  |  |
| Operatories |  |  |  |
| ☐ | Flush each waterline for the time your waterline protocol sets |  |  |
| ☐ | Check the chair, light, handpieces and suction work in each operatory |  |  |
| ☐ | Put fresh barriers on the light handles, chair controls and keyboards |  |  |
| ☐ | Check gloves, masks, eye protection and gowns are stocked |  |  |
| ☐ | Check today’s instrument packs are sealed, dated and their indicators have changed |  |  |
| Emergency equipment |  |  |  |
| ☐ | Check the emergency kit is complete and sealed |  |  |
| ☐ | Check the oxygen cylinder is full enough, with its mask and tubing |  |  |
| ☐ | Check the AED status light is green |  |  |
| Reception |  |  |  |
| ☐ | Take the phones off divert and check messages |  |  |
| ☐ | Check today’s schedule, and that lab work for today’s patients has arrived |  |  |
| ☐ | Flag patients due to update their medical history |  |  |
| ☐ | Confirm tomorrow’s appointments |  |  |

| Completed by |  | Checked by (manager) |  | Date |  |
| --- | --- | --- | --- | --- | --- |
| Notes |  |  |  |  |  |

# Closing checklist

After the last patient

| Done | Task | Time | Initials |
| --- | --- | --- | --- |
| Operatories |  |  |  |
| ☐ | Remove barriers, then clean and disinfect every surface touched |  |  |
| ☐ | Flush the waterlines and run cleaner through the suction lines Use the products and times the makers set. |  |  |
| ☐ | Take all used instruments to the sterilization area |  |  |
| ☐ | Turn off the chair, light and delivery unit |  |  |
| Sterilization |  |  |  |
| ☐ | Clean and process every used instrument |  |  |
| ☐ | Record every sterilizer cycle in the sterilization log |  |  |
| ☐ | Empty, clean and dry the ultrasonic cleaner |  |  |
| ☐ | Turn off the sterilizer once the last cycle is done |  |  |
| Waste |  |  |  |
| ☐ | Seal full sharps containers and store them for collection |  |  |
| ☐ | Bag and store clinical waste in the locked waste area |  |  |
| Reception |  |  |  |
| ☐ | Reconcile the day’s payments with the practice software |  |  |
| ☐ | Check tomorrow’s schedule and that its lab work is booked in |  |  |
| ☐ | Put the phones on divert or the after-hours message |  |  |
| Lock-up |  |  |  |
| ☐ | Turn off the compressor and suction, and drain them if required |  |  |
| ☐ | Lock the medicines cupboard |  |  |
| ☐ | Turn off the lights, lock all doors and set the alarm |  |  |

| Completed by |  | Checked by (manager) |  | Date |  |
| --- | --- | --- | --- | --- | --- |
| Notes |  |  |  |  |  |