SOP TEMPLATE

# Lockout tagout isolation procedure

Check this template against your local regulations and your own policies before you use it.

| SOP no. |  | Version |  | Effective date |  |
| --- | --- | --- | --- | --- | --- |
| Owner |  | Approver |  | Review date |  |

# Purpose

To make sure no one works on equipment that could be energised or start up.

# Scope

All work on electrical circuits, machinery, plumbing, gas and pressurised systems.

# Responsibilities

| Role | Responsibility |
| --- | --- |
| Tradesperson | Isolates, locks, tags and tests before work, and removes only their own lock. |
| Site contact | Is told about the isolation and keeps others away from it. |

# What you need

- Personal locks and danger tags

- A tested voltage tester or the right test instrument

# Procedure

- Identify every energy source

Find all power, gas, water, pressure and stored energy that could reach the work.

- Tell others

Tell the site contact and anyone affected what will be isolated and for how long.

- Isolate

Switch off and isolate at the isolation point, not just at the switch.

- Lock and tag

Fit your personal lock and a danger tag with your name, the date and the work.

- Test for dead

Test that the equipment is dead or depressurised, with a tester you have checked works.

Caution: Treat everything as live until you have tested it yourself.

- Do the work

Release stored energy, such as capacitors or springs, before starting.

- Restore safely

Remove tools, refit covers, tell others, remove your lock and tag, and test that it works.

# Records

- The job record of the isolation.

# Related documents

- Daily pre-start safety checklist

# Revision history

| Version | Date | What changed | Approved by |
| --- | --- | --- | --- |